The PEDROLLO CP range are non self-priming closed impeller pumps with integral motors. They are efficient, quiet and rated for continuous duty making them suitable for water boosting and transfer in a variety of domestic, industrial and agricultural applications. They are also particularly suited to automatic pressurised supply using either a surge tank or 'Presscontrol' unit. Pump construction material is cast iron for the casing and brass alloy for impellers except Cpm130 which uses techno plastic and CPm150 which uses stainless steel.

MOTOR
Pumps are close coupled to a Pedrollo brushless TEFC induction motor designed for continuous operation. Single phase motors are provided with a thermal cutout in the windings to protect against electric and mechanical overload and can be connected directly to the mains power supply through a 10Amp fuse or MCB. Three phase motors require remote DOL starter protection according to local regulations.

PUMP DATA

| Model | Voltage (V) | Power (kW | Current (A) | DN1 | DN2 | Dimensions (mm) |  |  |  | Weight (kg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | L | H | H1 | W |  |
| CPm 130 | 1x240 | 0.37 | 2.9 | 1” | 1” | 258 | 205 | 82 | 165 | 8 |
| CPm 150 |  | 0.75 | 5.4 |  |  | 286 | 240 | 92 | 190 | 12 |
| CPm 158 |  |  | 5.8 |  |  | 286 | 240 | 92 | 190 | 12 |
| CPm 158 Hydrofresh |  |  |  |  |  | 550 | 600 | - | 225 | 20 |
| CPm 170 |  | 1.1 | 7.2 | 1¼” |  | 367 | 260 | 110 | 206 | 18 |
| CPm 190 |  | 1.5 | 10 |  |  | 364 | 290 | 115 | 242 | 22 |
| CPm 200 |  | 2.2 | 11.7 |  |  | 384 |  |  |  | 24 |
| CP 200 | 3x415 |  | 5.2 |  |  |  |  |  |  | 24 |